CIVE 625 – Quantitative Ecohydrology

Homework 1 (Due Feb 2nd 2024)

Problem 1 (25 pt) Assume the air temperature to be 32 ºC and the relative humidity 50%, the air pressure is at 95 kPa. Compute the following variables:

- Dew point temperature

20.27 C

- Wet bulb temperature if the effective psychrometric constant * is 66 Pa/ºC

29.39 C

- Specific humidity in g/kg

0.0153 g/kg

- Mixing ratio in g/kg

0.0156 g/kg

- If the air cools at night to 18 ºC, will fog form near the surface?

- Yes because 18 C is below the dew point temperature of 20.3 C.

Problem 2 (15 pt) Using the empirical formula for esat given as Equation (2.17) in Terrestrial Hydrometeorology, derive the expression for the dew point temperature for air with vapor pressure e given in the book as Equation (2.21). [Note: the most common error made when using this formula is to use logarithms to base 10 rather than natural logarithms. Be careful not to make this mistake.]

Problem 3 (25 pt) Compute the daily extraterrestrial solar radiation Sod [W/m2] in El Paso, TX and for Fort Collins, CO, for February 7. Repeat your computations for every DOY (don’t report!) and produce a time series plot of daily extraterrestrial solar radiation in function of DOY with both cities included.

February 7th:

Fort Collins Radiation: 210.66 W/m^2

El Paso Radiation: 269.70 W/m^2

Problem 4 (20 pt) The following data are averages for a typical summer day in a temperate climate: air temperature, Ta = 17.94ºC; relative humidity, 66%; and incoming, short-wave radiation, Sin = 468 cal cm-2 d-1. Calculate the net radiation, Rn (in W m-2), for a surface covered with short vegetation. Assume that the daily average of the surface temperature, Ts, is the same as the air temperature, and that cloudiness does not affect the long-wave radiation, when it is derived from Ta.

Net Radiation: 72 W/m2

Problem 5 (15 pt) Consider the following observations: net radiation Rn = 200 Wm-2; heat flux into the ground, G = 40 Wm-2; and evaporation rate, E = 5 10-8 ms-1.

- Calculate the turbulent sensible heat flux, H, in Wm-2

  - Sensible Heat Flux: 160 W/m2

- Was the atmosphere stable or unstable? Why?

  - The atmosphere is unstable because the net radiation is positive, which means the surface is warmer than the air above it. This will cause the air to rise and the atmosphere to be unstable.

- Was the soil warming up or cooling down? Why?

  - The soil is warming up because the net radiation is positive and the heat flux into the ground is positive.